DTMF Signal Detection

README & Technical Documentation

Python Implementation using Goertzel Algorithm

# Overview

This script implements DTMF (Dual-Tone Multi-Frequency) signal detection in Python. It generates, processes, and decodes the tones used by telephone keypads by applying the Goertzel algorithm — a computationally efficient method for detecting specific frequencies in a signal.

# Dependencies

The following Python libraries are required:

- numpy — Numerical operations and array handling

- matplotlib — Signal visualization and plotting

Install via pip:

pip install numpy matplotlib

# How It Works

## DTMF Frequency Map

DTMF encodes each digit as a combination of one low-frequency and one high-frequency tone:

| Parameter | Value |
| --- | --- |
| Sampling Frequency (fs) | 8000 Hz |
| Low Frequencies | 697, 770, 852, 941 Hz |
| High Frequencies | 1209, 1336, 1477 Hz |
| Tone Duration | 0.3 seconds |
| Silence Gap | 0.05 seconds |
| Noise Level | 0.2 (Gaussian) |

## Goertzel Algorithm

The Goertzel algorithm is used to efficiently compute the power of specific frequency components in a signal — much more efficient than a full FFT when only a small number of frequencies need to be checked.

Key steps:

- Compute a target bin k from the desired frequency and signal length

- Apply a second-order IIR filter iteratively over the signal

- Compute the output power from the final two filter states

## Detection Flow

- A DTMF tone sequence is synthesized for each digit in the input string

- White Gaussian noise is added to simulate a real phone channel

- The signal is split into fixed-size frames matching the tone duration

- The Goertzel algorithm is run for each DTMF frequency on every frame

- The dominant low and high frequencies are identified per frame

- Each (low, high) frequency pair is mapped back to its corresponding digit

# Code Structure

| Function / Variable | Description |
| --- | --- |
| goertzel(signal, target_freq, fs) | Computes signal power at a given frequency using the Goertzel algorithm |
| detect_dtmf(signal) | Detects the dominant low and high DTMF frequency in a signal frame |
| dtmf_map | Dictionary mapping (low_freq, high_freq) tuples to digit characters |
| digits | Input phone number string to encode and decode |
| signal | Synthesized audio signal including tones, silence gaps, and noise |
| frame_size / step | Frame window size and step for sliding detection |

# Usage

To run the script, simply execute it with Python:

python dtmf_detection.py

To detect a different phone number, modify the digits variable in the script:

digits = "08242474040"  # Replace with your target number

# Expected Output

The script produces two outputs:

- A matplotlib plot showing the first 2000 samples of the generated DTMF signal

- A console printout of the detected phone number, e.g.:

Detected Number: 08242474040

# Tuning & Notes

Detection Threshold:

The threshold variable in detect_dtmf() (default: 100) controls sensitivity. Lower it for noisy environments; raise it to reduce false positives.

Limitations:

- Currently supports digits 0-9 only (no *, # keys)

- Frame-based detection may miss tones at boundaries if timing is off

- Noise level is fixed at 0.2 and can be adjusted in the signal generation section

DTMF Signal Detection — Python Implementation